# VGA Sudoku Game

FPGA Implementation for Basys3 Board

Project Documentation and Technical Report

CSCEA 342 - Digital Circuits

University of Alaska Anchorage

Engineers:

Gwendolyn Beecher

Constantine Hinds

December 2025

License: GNU General Public License v3.0

This report was generated by Claude AI (Anthropic)

based on project files and a basic draft provided by the authors.

# 1. Project Overview

This project implements a fully playable Sudoku game on the Basys3 FPGA development board with VGA display output. The system features a 640×480 VGA display showing a 9×9 Sudoku grid, cursor-based navigation, number entry with preview, puzzle selection, real-time correctness checking, and win/lose state detection.

## 1.1 Key Features

- VGA 640×480 @ 60Hz display with centered 360×360 pixel Sudoku grid

- Three selectable puzzles with on-screen game selector UI (left panel)

- Two-mode input system: MOVE mode for cursor navigation, NUMBER mode for digit entry

- Flashing cursor with number preview in NUMBER mode

- Seven-segment display showing cursor position (X,Y), edit indicator, and current/selected value

- Fixed cell protection (pre-filled puzzle values cannot be modified)

- Real-time correctness checking via SW2 (cells turn green/red)

- Right-side UI panel with WIN, LOSE, and ENTER 2 status boxes

- Automatic puzzle reload when switching between puzzles

## 1.2 Hardware Platform

Target Board: Digilent Basys3 (Xilinx Artix-7 XC7A35T)

Clock: 100 MHz system clock, divided to 25 MHz for VGA pixel clock

Display: VGA output at 640×480 resolution

Input: 5 push buttons (Center, Up, Down, Left, Right), 16 switches

Output: 4-digit seven-segment display, VGA connector

# 2. System Architecture

The system follows a simplified modular design with the game engine handling all puzzle storage, cursor control, and correctness checking internally using arrays rather than separate RAM modules.

## 2.1 Module Hierarchy

top.sv (Top-level module)

├── clock_divider.sv - 100MHz → 25MHz

├── vga_controller.sv - VGA timing generation

├── conditioner.sv (×5) - Button debounce/one-pulse

├── sudoku_engine.sv - Game logic (integrated design)

├── sudoku_draw.sv - VGA pixel rendering

│   ├── digit_font.sv - 8×12 digit bitmaps

│   └── letter_font.sv - 8×12 letter bitmaps

└── sevenseg_sudoku.sv - 7-segment controller

└── sevenseg.sv - Low-level 7-seg driver

## 2.2 Data Flow

1. Puzzle Loading: On reset or puzzle change, sudoku_engine reads from internal ROM (puzzles.mem via $readmemh) and populates internal grid, solution, and fixed_mask arrays.

2. Gameplay: sudoku_engine handles cursor movement and number entry directly, updating internal arrays.

3. Display: sudoku_draw reads grid_out and fixed_mask_out (combinationally exposed from engine) to render pixels in real-time with zero latency.

4. Correctness: When SW2 is high, cell_match array indicates per-cell correctness; check_win/check_lose signals drive UI.

## 2.3 State Machines

### 2.3.1 Game Engine State Machine (sudoku_engine.sv)

The game engine operates with a four-state FSM:

| State | Description | Transition |
| --- | --- | --- |
| S_LOAD_INIT | Load puzzle grid from ROM | → S_LOAD_SOL after 81 cycles |
| S_LOAD_SOL | Load solution grid from ROM | → S_PLAY after 81 cycles |
| S_PLAY | Active gameplay (cursor, entry) | → S_CHECK_WIN on ENTER |
| S_CHECK_WIN | Verify solution correctness | → S_PLAY after 81 cycles |

### 2.3.2 Mode State Machine (top.sv)

The top-level module implements a two-state mode machine for user input:

| Mode | Button Functions | Cursor Display |
| --- | --- | --- |
| MODE_MOVE | U/D/L/R: cursor movement, C: switch to NUMBER | Solid (always visible) |
| MODE_NUMBER | U/D: increment/decrement number, C: enter value | Flashing (~0.67s period) |

### 2.3.3 Seven-Segment Display (sevenseg_sudoku.sv)

| Digit | MODE_MOVE | NUMBER (editable) | NUMBER (fixed) |
| --- | --- | --- | --- |
| 0 (left) | X position (0-8) | X position (0-8) | X position (0-8) |
| 1 | Y position (0-8) | Y position (0-8) | Y position (0-8) |
| 2 | Blank | 'E' (Edit indicator) | Blank |
| 3 (right) | Current cell value | Flashing selected number | Current cell value |

# 3. Module Documentation

## 3.1 top.sv - Top-Level Module

Authors: Gwendolyn Beecher, Constantine Hinds

Lines: 278

Purpose: Top-level integration module that instantiates and connects all subsystems.

Key Features:

- Switch synchronizers for stable puzzle selection (SW0, SW1)

- Correctness mode enable via SW2 (check_enable signal)

- Soft reset generation when puzzle selection changes

- Mode logic (MOVE vs NUMBER) with center button toggle

- Cursor position tracking for automatic number selection in NUMBER mode

- Cursor flashing logic using bit 26 of a counter (~0.67s period)

## 3.2 sudoku_engine.sv - Game Engine

Authors: Gwendolyn Beecher, Constantine Hinds

Lines: 221

Purpose: Central game engine that handles puzzle loading, cursor movement, number entry, and correctness checking.

Architecture:

- Internal storage arrays: grid[0:8][0:8], solution[0:8][0:8], fixed_mask[0:8][0:8]

- Direct ROM loading via $readmemh("puzzles.mem", puzzle_rom)

- Nibble extraction for cell values from packed ROM data

- Combinational grid exposure (always_comb) for zero-latency VGA display

- Real-time cell_match computation when check_enable is active

- Integrated cursor movement with wrap-around (0↔8)

## 3.3 sudoku_draw.sv - VGA Renderer

Authors: Gwendolyn Beecher, Constantine Hinds

Lines: 453

Purpose: Combinational pixel color generation for VGA display including game board, UI panels, and correctness overlay.

Layout:

- Sudoku board: 360×360 pixels (9×9 cells, 40px each), centered at (140, 60)

- Left panel: Game selector boxes (DFLT, GAME 0, GAME 1) at x=20

- Right panel: Status boxes (WIN, LOSE, ENTER 2) at x=550

Color Palette:

| Element | Color | Hex | RGB |
| --- | --- | --- | --- |
| Background | Dark blue-gray | 0x334 | (51, 51, 68) |
| Pre-filled cell | Light teal | 0xABB | (170, 187, 187) |
| Editable cell | Off-white | 0xFFE | (255, 255, 238) |
| Cursor | Pink | 0xF59 | (255, 85, 153) |
| Correct (SW2) | Green | 0x4F4 | (68, 255, 68) |
| Incorrect (SW2) | Red | 0xF44 | (255, 68, 68) |
| ENTER 2 box | Yellow | 0xDD7 | (221, 221, 119) |

## 3.4 vga_controller.sv - VGA Timing

Author: Gwendolyn Beecher

Lines: 67

Purpose: Standard VGA timing generator for 640×480 @ 60Hz.

| Parameter | Horizontal | Vertical |
| --- | --- | --- |
| Active | 640 pixels | 480 lines |
| Front Porch | 16 pixels | 10 lines |
| Sync Pulse | 96 pixels | 2 lines |
| Back Porch | 48 pixels | 33 lines |
| Total | 800 pixels | 525 lines |

## 3.5 Supporting Modules

| Module | Lines | Description |
| --- | --- | --- |
| clock_divider.sv | 26 | Divides 100MHz to 25MHz for VGA pixel clock |
| conditioner.sv | 38 | Button debounce (1.5M cycles) + one-pulse generation |
| digit_font.sv | 168 | 8×12 bitmap font for digits 1-9 |
| letter_font.sv | 266 | 8×12 bitmap font for letters (G,A,M,E,D,F,L,T,S,W,I,N,R,O) and digits 0-2 |
| sevenseg.sv | 75 | Low-level 7-segment driver with multiplexed refresh |
| sevenseg_sudoku.sv | 75 | Sudoku-specific display logic (position, edit, value) |
| puzzles.mem | 7 | Hex puzzle data: 6 rows × 81 nibbles (3 puzzles × init+solution) |
| Basys3_Master.xdc | 295 | Pin constraints for VGA, buttons, switches, 7-segment |

# 4. Removed Modules

The following modules were removed from the original design due to unnecessary complexity introduced by the RAM-based architecture with shadow registers. The simplified design uses internal arrays instead, which provides immediate VGA display updates without the synchronization delays inherent in the RAM/shadow register approach.

## 4.1 Modules Removed

| Module | Reason for Removal |
| --- | --- |
| sudoku_loader.sv | Loading logic integrated into sudoku_engine.sv S_LOAD_INIT/S_LOAD_SOL states |
| sudoku_gameplay.sv | Cursor and number entry logic integrated into sudoku_engine.sv S_PLAY state |
| sudoku_checker.sv | Verification logic integrated into sudoku_engine.sv S_CHECK_WIN state |
| shadow_register.sv | Replaced by combinational always_comb block exposing grid arrays directly |
| ram.sv | Replaced by internal logic arrays (grid, solution, fixed_mask) |
| puzzle_rom.sv | Replaced by direct $readmemh in sudoku_engine.sv |
| sudoku_games.sv | Hardcoded puzzles unused; puzzles.mem is primary data source |

## 4.2 Technical Justification

The original RAM-based architecture caused a VGA display delay bug:

- In NUMBER mode, the seven-segment display updated immediately when incrementing numbers

- However, the VGA display lagged until the value was actually written to RAM

- Root cause: shadow_register only updated on RAM write, not on preview_number change

- The fix required either modifying display priority logic OR simplifying the architecture

The simplified architecture resolves this by:

- Using internal arrays instead of RAM modules

- Exposing grid_out via combinational always_comb (zero latency)

- Eliminating RAM arbitration and shadow register synchronization

- Reducing total module count from 19 to 12 files

# 5. AI Usage Acknowledgment

In accordance with academic integrity requirements, the following documents all instances where AI assistance (Claude by Anthropic) was used during this project's development.

## 5.1 Report Generation

This report document was generated by Claude AI (Anthropic) based on a basic draft outline and the complete project source files provided by the authors. The AI organized the content, wrote technical descriptions, formatted tables, and structured the document according to the authors' specifications.

## 5.2 Summary of AI-Assisted Development Tasks

- Architecture Simplification: Proposed removing RAM/shadow register architecture to fix VGA display delay bug

- Engine Consolidation: Integrated loader, gameplay, and checker logic into single sudoku_engine module

- Correctness Checking: Added check_enable, check_win, check_lose, and cell_match outputs for SW2 feature

- UI Enhancement: Added right-side status boxes (WIN, LOSE, ENTER 2) to sudoku_draw

- Font Extension: Added letters I, N, R, O to letter_font.sv for WIN/LOSE/ENTER display

- Cursor Tracking: Added automatic number selection update when cursor moves in NUMBER mode

- Seven-Segment Integration: Guided integration of flashing behavior and mode-aware display

- General Debugging: Various bug fixes including ROM loading, display timing, and state machine transitions

- Code Documentation: Added comments and corrected author name spelling (Hinds)

- Report Generation: Created this technical documentation

## 5.3 Human Contributions

The following work was performed primarily by the human engineers:

- Original project concept and requirements definition

- Initial architecture design and module planning

- VGA controller timing implementation (Gwendolyn)

- Digit and letter font bitmap design (Gwendolyn)

- Puzzle selection and puzzle data (Gwendolyn)

- Clock divider implementation (Gwendolyn)

- Button conditioner/debouncer logic

- Hardware testing and validation on Basys3

- All design decisions and architectural choices

- Final integration and verification

# 6. License

This project is licensed under the GNU General Public License v3.0.

GNU General Public License v3.0

Copyright (C) 2025 Gwendolyn Beecher, Constantine Hinds

This program is free software: you can redistribute it and/or modify it under the terms of the GNU General Public License as published by the Free Software Foundation, either version 3 of the License, or (at your option) any later version.

This program is distributed in the hope that it will be useful, but WITHOUT ANY WARRANTY; without even the implied warranty of MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU General Public License for more details.

# 7. Complete File Listing

| File | Lines | Description |
| --- | --- | --- |
| top.sv | 278 | Top-level integration module |
| sudoku_engine.sv | 221 | Game state machine and logic |
| sudoku_draw.sv | 453 | VGA pixel rendering with UI panels |
| letter_font.sv | 266 | 8×12 letter bitmaps |
| digit_font.sv | 168 | 8×12 digit bitmaps |
| sevenseg_sudoku.sv | 75 | Sudoku-specific 7-seg logic |
| sevenseg.sv | 75 | Low-level 7-seg driver |
| vga_controller.sv | 67 | VGA timing generation |
| conditioner.sv | 38 | Button debounce/one-pulse |
| clock_divider.sv | 26 | 100MHz → 25MHz |
| puzzles.mem | 7 | Puzzle data file |
| Basys3_Master.xdc | 295 | Pin constraints |

--- End of Document ---