# VGA Sudoku Game

FPGA Implementation for Basys3 Board

Project Documentation and Technical Report

CSCEA 342 - Digital Circuits

University of Alaska Anchorage

Engineers:

Gwendolyn Beecher

Constantine Hinds

December 2025

License: GNU General Public License v3.0

This report was generated by Claude AI (Anthropic)

based on project files and a basic draft provided by the authors.

# Table of Contents

1. Project Overview

2. System Architecture

3. Module Documentation

4. AI Usage Acknowledgment

5. Professor-Provided Code

6. References and External Sources

7. License

8. File Listing

# 1. Project Overview

This project implements a fully playable Sudoku game on the Basys3 FPGA development board with VGA display output. The system features a 640×480 VGA display showing a 9×9 Sudoku grid, cursor-based navigation, number entry with preview, puzzle selection, and win/lose state detection.

## 1.1 Key Features

- VGA 640×480 @ 60Hz display with centered 360×360 pixel Sudoku grid

- Three selectable puzzles with on-screen game selector UI

- Two-mode input system: MOVE mode for cursor navigation, NUMBER mode for digit entry

- Flashing cursor with number preview in NUMBER mode

- Seven-segment display showing cursor position (X,Y), edit indicator, and current/selected value

- Fixed cell protection (pre-filled puzzle values cannot be modified)

- Solution checking with win/lose state detection

- Automatic puzzle reload when switching between puzzles

## 1.2 Hardware Platform

Target Board: Digilent Basys3 (Xilinx Artix-7 XC7A35T)

Clock: 100 MHz system clock, divided to 25 MHz for VGA pixel clock

Display: VGA output at 640×480 resolution

Input: 5 push buttons (Center, Up, Down, Left, Right), 16 switches

Output: 4-digit seven-segment display, VGA connector

# 2. System Architecture

The system follows a modular hierarchical design with clear separation between game logic, display rendering, and I/O handling.

## 2.1 Module Hierarchy

top.sv (Top-level module)

├── clock_divider.sv      - 100MHz → 25MHz

├── vga_controller.sv     - VGA timing generation

├── conditioner.sv (×5)   - Button debounce/one-pulse

├── sudoku_engine.sv      - Game state machine

│   ├── puzzle_rom.sv     - Puzzle storage

│   ├── ram.sv (×3)       - Grid, Solution, Mask RAMs

│   ├── shadow_register.sv - VGA display buffer

│   ├── sudoku_loader.sv  - Puzzle loading FSM

│   ├── sudoku_gameplay.sv - Cursor and number entry

│   └── sudoku_checker.sv - Win/lose detection

├── sudoku_draw.sv        - VGA pixel rendering

│   ├── digit_font.sv     - 8×12 digit bitmaps

│   └── letter_font.sv    - 8×12 letter bitmaps

└── sevenseg_sudoku.sv    - 7-segment controller

└── sevenseg.sv       - Low-level 7-seg driver

## 2.2 Data Flow

The system operates with the following data flow:

- Puzzle Loading: On reset or puzzle change, sudoku_loader reads from puzzle_rom and writes to Grid RAM, Solution RAM, Mask RAM, and Shadow Register

- Gameplay: sudoku_gameplay handles cursor movement and number entry, updating Grid RAM and Shadow Register

- Display: sudoku_draw reads from Shadow Register (9×9 arrays) to render pixels in real-time

- Checking: sudoku_checker compares Grid RAM against Solution RAM cell-by-cell

## 2.3 State Machines

### 2.3.1 Main Game Engine State Machine (sudoku_engine.sv)

The main game engine operates with a three-state FSM:

| State | Description | Button Input | Transition |
| --- | --- | --- | --- |
| S_LOADING | Load puzzle data into RAMs | None (automatic) | → S_PLAY when load_done |
| S_PLAY | Active gameplay state | U/D/L/R: cursor move Center: mode toggle/enter | No transition to S_CHECKING |
| S_CHECKING | Verify solution correctness | None (automatic) | → S_PLAY when check_done |

Note: The current implementation does not have an active transition from S_PLAY to S_CHECKING. The enter_evt signal exists but the checker is not triggered during normal gameplay. This means the win/lose checking functionality is not accessible to the player.

### 2.3.2 Puzzle Loader State Machine (sudoku_loader.sv)

| State | Description | Duration | Transition |
| --- | --- | --- | --- |
| S_IDLE | Waiting for start signal | Until start_load | → S_LOAD_INIT |
| S_LOAD_INIT | Write grid values + fixed mask to RAMs and shadow register | 81 clock cycles | → S_LOAD_SOL |
| S_LOAD_SOL | Write solution values to Solution RAM | 81 clock cycles | → S_DONE |
| S_DONE | Signal completion | 1 clock cycle | → S_IDLE |

### 2.3.3 Solution Checker State Machine (sudoku_checker.sv)

| State | Description | Outputs |
| --- | --- | --- |
| S_IDLE | Wait for start_check pulse | checking=0, game_won/lost hold previous |
| S_CHECK | Compare grid_value vs solution_value for each cell (0-80) | checking=1, won_reg=1 until mismatch |
| S_DONE | Signal check_done pulse | check_done=1, game_won/lost finalized |

### 2.3.4 Mode State Machine (top.sv)

The top-level module implements a two-state mode machine for user input:

| Mode | Button Functions | Cursor Display | Transition |
| --- | --- | --- | --- |
| MODE_MOVE | U: cursor up D: cursor down L: cursor left R: cursor right C: switch to NUMBER mode | Solid (always visible) | Center button → MODE_NUMBER |
| MODE_NUMBER | U: increment number (1→9→1) D: decrement number (9→1→9) L: (no action) R: (no action) C: enter value & switch to MOVE | Flashing (~0.67s period) | Center button → MODE_MOVE |

### 2.3.5 Seven-Segment Display Logic (sevenseg_sudoku.sv)

The seven-segment display shows different information based on mode:

| Digit | MODE_MOVE | MODE_NUMBER (editable) | MODE_NUMBER (fixed) |
| --- | --- | --- | --- |
| 0 (left) | X position (0-8) | X position (0-8) | X position (0-8) |
| 1 | Y position (0-8) | Y position (0-8) | Y position (0-8) |
| 2 | Blank | 'E' (Edit indicator) | Blank |
| 3 (right) | Current cell value (or blank if 0) | Flashing selected number | Current cell value |

### 2.3.6 VGA Display Pipeline

The VGA display operates as a purely combinational pipeline synchronized to the pixel clock:

VGA Timing (25 MHz pixel clock):

┌─────────────────────────────────────────────────────────────┐

│  vga_controller.sv generates:                               │

│    • x, y coordinates (0-639, 0-479 in active region)       │

│    • hsync, vsync signals                                   │

│    • video_on flag                                          │

└─────────────────────────────────────────────────────────────┘

↓

┌─────────────────────────────────────────────────────────────┐

│  sudoku_draw.sv determines pixel color based on:            │

│    • Board region: grid lines, cell backgrounds, digits     │

│    • Game selector region: boxes, labels, highlight         │

│    • Background: default color outside board                │

│                                                             │

│  Inputs read combinationally:                               │

│    • grid_vals[0:8][0:8] from shadow_register               │

│    • fixed_mask[0:8][0:8] from shadow_register              │

│    • cursor_x, cursor_y from gameplay                       │

│    • flash_state from top.sv counter                        │

│    • preview_number (in NUMBER mode)                        │

└─────────────────────────────────────────────────────────────┘

↓

┌─────────────────────────────────────────────────────────────┐

│  Output to VGA DAC:                                         │

│    • vgaRed[3:0], vgaGreen[3:0], vgaBlue[3:0]              │

│    • Blanked (all 0) when video_on = 0                      │

└─────────────────────────────────────────────────────────────┘

### 2.3.7 Value Entry Flow

When entering a value in NUMBER mode:

1. User in MODE_MOVE, cursor on desired cell

↓

2. Press CENTER → switch to MODE_NUMBER

• Cursor starts flashing

• Seven-segment shows 'E' and flashing '1'

• Preview number appears in cursor cell (if empty)

↓

3. Press UP/DOWN to cycle selected_number (1-9)

• Preview updates on VGA

• Seven-segment digit 3 flashes new number

↓

4. Press CENTER to confirm entry

• If cell is NOT fixed (cell_is_editable = 1):

- grid_we = 1, grid_din = selected_number

- shadow_we = 1, shadow_grid_din = selected_number

- Value written to Grid RAM and Shadow Register

• If cell IS fixed:

- Write signals remain 0

- No change occurs

• Mode returns to MODE_MOVE

• Cursor stops flashing

# 3. Module Documentation

## 3.1 top.sv - Top-Level Module

Authors: Gwendolyn Beecher, Constantine Hinds

Purpose: Top-level integration module that instantiates and connects all subsystems.

Key Features:

- Switch synchronizers for stable puzzle selection

- Soft reset generation when puzzle changes

- Mode logic (MOVE vs NUMBER) with center button toggle

- Cursor flashing logic using bit 26 of a counter (~0.67s period)

- Command generation based on current mode

## 3.2 sudoku_engine.sv - Game Engine

Authors: Gwendolyn Beecher, Constantine Hinds

Purpose: Central game state machine that coordinates all game logic submodules.

Architecture:

- Three RAM instances: Grid (player values), Solution (correct answers), Mask (fixed cells)

- Shadow Register for VGA display (parallel access to all 81 cells)

- RAM multiplexing based on game state (loader/gameplay/checker)

- Input event latching for stable button handling

## 3.3 shadow_register.sv - Display Buffer

Authors: Gwendolyn Beecher, Constantine Hinds

Purpose: Provides parallel access to all 81 grid cells for VGA rendering without RAM contention.

Implementation:

- 81-entry case statement for address-to-array mapping

- Dual 9×9 arrays: grid_shadow (cell values) and fixed_mask_shadow (editability)

- Direct wire assignments for output (combinational read)

Design History: The shadow register was proposed by AI to address a synthesis bug. The original approach attempted to use a Moore-style state machine with RAM, but this proved impractical for synthesis. AI proposed the shadow register architecture as a fix, and assisted in converting the design into the explicit case statement structure for synthesis compatibility.

## 3.4 sudoku_loader.sv - Puzzle Loader

Authors: Gwendolyn Beecher, Constantine Hinds

Purpose: Sequential loading of puzzle data from ROM into RAMs and shadow register.

State Machine:

- S_IDLE: Waiting for start_load pulse

- S_LOAD_INIT: Write initial grid values and fixed mask (81 cycles)

- S_LOAD_SOL: Write solution values (81 cycles)

- S_DONE: Signal completion, return to idle

## 3.5 sudoku_gameplay.sv - Player Interaction

Authors: Gwendolyn Beecher, Constantine Hinds

Purpose: Handles cursor movement and number entry during gameplay.

Features:

- Cursor position tracking with wrap-around (0→8→0)

- Linear address calculation: cursor_adr = cursor_y × 9 + cursor_x

- Fixed cell protection (writes blocked when cell_fixed is true)

- Simultaneous write to Grid RAM and Shadow Register

## 3.6 sudoku_checker.sv - Solution Verification

Authors: Gwendolyn Beecher, Constantine Hinds

Purpose: Compares player grid against solution to determine win/lose state.

Algorithm:

- Initialize won_reg = 1 (assume win)

- Iterate through cells 0-80

- If any grid_value ≠ solution_value: set won_reg = 0, lost_reg = 1

- Signal check_done when counter reaches 80

## 3.7 puzzle_rom.sv - Puzzle Storage

Authors: Gwendolyn Beecher, Constantine Hinds

Purpose: ROM containing three Sudoku puzzles with initial values and solutions.

Implementation:

- File-based loading from puzzles.mem with $fopen/$fgetc

- Hardcoded fallback puzzles if file read fails

- ASCII digit conversion ('0'-'9' → 0-9)

Note: AI assistance was used to debug issues with puzzles.mem not reading correctly and not displaying on screen.

## 3.8 sudoku_draw.sv - VGA Renderer

Authors: Gwendolyn Beecher, Constantine Hinds

Purpose: Combinational pixel color generation for VGA display.

Layout Constants:

- CELL_SIZE = 40 pixels (360×360 total board)

- BOARD_X0 = 140, BOARD_Y0 = 60 (centered on 640×480)

- Game selector: 20px from left, 70px wide, 42px tall per box

Color Palette:

| Element | Color | Hex | RGB |
| --- | --- | --- | --- |
| Background | Dark blue-gray | 0x334 | (51, 51, 68) |
| Pre-filled cell | Light teal | 0xABB | (170, 187, 187) |
| Editable cell | Off-white | 0xFFE | (255, 255, 238) |
| Cursor | Pink | 0xF59 | (255, 85, 153) |

## 3.9 vga_controller.sv - VGA Timing

Author: Gwendolyn Beecher

Purpose: Standard VGA timing generator for 640×480 @ 60Hz.

Timing Parameters:

| Parameter | Horizontal | Vertical |
| --- | --- | --- |
| Active | 640 pixels | 480 lines |
| Front Porch | 16 pixels | 10 lines |
| Sync Pulse | 96 pixels | 2 lines |
| Back Porch | 48 pixels | 33 lines |
| Total | 800 pixels | 525 lines |

## 3.10 sevenseg_sudoku.sv - Seven-Segment Display

Authors: Gwendolyn Beecher, Constantine Hinds

Purpose: Displays cursor position and value with mode-aware flashing.

Display Format:

- Digit 0 (leftmost): X position (0-8)

- Digit 1: Y position (0-8)

- Digit 2: 'E' if in NUMBER mode and cell is editable, else blank

- Digit 3: Current value or flashing selected number

Note: AI assistance was used to integrate the flashing behavior and adapt the professor-provided seven-segment display module.

# 4. AI Usage Acknowledgment

In accordance with academic integrity requirements, the following documents all instances where AI assistance (Claude by Anthropic) was used during this project's development.

## 4.1 Report Generation

This report document was generated by Claude AI (Anthropic) based on a basic draft outline and the complete project source files provided by the authors. The AI organized the content, wrote technical descriptions, formatted tables, and structured the document according to the authors' specifications.

## 4.2 Summary of AI-Assisted Development Tasks

- Shadow Register Proposal: AI proposed the shadow register architecture to fix a synthesis bug. The original design attempted to use a Moore-style state machine with RAM which proved impractical

- RAM Integration: Integrated the professor-provided ram.sv module for shadow registers, adapting it to work with the 9×9 grid structure

- Case Statement Conversion: Converted the shadow register design into explicit 81-entry case statements and wire assignments for synthesis compatibility

- CI/CD Pipeline Creation: Created GitHub Actions workflow for automated syntax, smoke, and semantic testing of SystemVerilog code

- Cross-Module RAM Integration: Integrated RAM modules across all code sections (loader, gameplay, checker, engine)

- Engine Modularization: Broke the monolithic sudoku_engine into smaller, focused submodules (loader, gameplay, checker)

- Puzzle ROM Debugging: Fixed bug where puzzles.mem would not read correctly or display on screen

- Seven-Segment Integration: Guided integration of seven-segment display with flashing number entry feedback

- Display Adaptation: Adapted professor-provided seven-segment module to show correct Sudoku-specific outputs

- General Bug Fixes: Various debugging and bug fixes throughout development

- Code Documentation: Added comments to code for clarity

- Repetitive Modifications: Made consistent changes across multiple files (e.g., correcting name spelling)

- Win/Lose State Machine Debugging: Step-by-step review and debugging of the sudoku_checker state machine

- Integration Assistance: Integrated Gwen's changes in top.sv (showing diff), fixed integration bugs

- Structure Simplification: Proposed simplified architectural structures

- Synthesis/Bitstream Debugging: Debugged synthesis, integration, and bitstream generation errors

- Testbench Adaptation: Adapted testbenches and updated test vectors (currently non-operational)

- Code Cleanup: Cleaned up overall code structure and organization

## 4.3 Human Contributions

The following work was performed primarily by the human engineers:

- Original project concept and requirements definition

- Initial architecture design and module planning

- VGA controller timing implementation (Gwendolyn)

- Digit and letter font bitmap design (Gwendolyn)

- Puzzle selection and hardcoded puzzle data (Gwendolyn)

- Clock divider implementation (Gwendolyn)

- Button conditioner/debouncer logic

- Hardware testing and validation on Basys3

- All design decisions and architectural choices

- Final integration and verification

# 5. Professor-Provided Code

The following code was provided by the course professor. Some modules were used as-is, while others were adapted for this project's specific requirements.

## 5.1 ram.sv - Parameterized RAM Module

Source: Assignment 5 Starter Code (sevenSegmentRamAssignmentHandout)

Status: Used as-is (unmodified)

Usage: Instantiated three times in sudoku_engine.sv for Grid RAM, Solution RAM, and Mask RAM

Complete Source Code:

module ram # (parameter N = 4, M = 16)

(input      logic           clk,        //clock

input       logic           we,         //write enable

input       logic [N-1:0]   adr,        //address

input       logic [M-1:0]   din,        //data in

output  logic [M-1:0]   dout);      //data out

logic [M-1:0] mem [2**N - 1:0];          //array of bits

always @ (posedge clk)

if (we) mem [adr] <= din;

// if write enabled, then assign din to a row in memory

assign dout = mem[adr];                 //read memory

endmodule

Instantiation in Project:

// Grid RAM: 7-bit address (0-127), 4-bit data (0-15)

ram #(.N(7), .M(4)) GRID_RAM (...);

// Solution RAM: Same parameters

ram #(.N(7), .M(4)) SOLUTION_RAM (...);

// Mask RAM: 7-bit address, 1-bit data (boolean)

ram #(.N(7), .M(1)) MASK_RAM (...);

## 5.2 sevenseg.sv - Seven-Segment Display Driver

Source: Assignment 5 Starter Code (sevenSegmentRamAssignmentHandout)

Status: Modified

Modifications:

- Changed input from 16-bit displayed_number to four separate 4-bit digit inputs (digit_0 through digit_3)

- Removed division/modulo operations for digit extraction

- Added support for 'E' character (4'hE) for edit mode indicator

- Added blank display code (4'hF) for unused digits

- Corrected cathode patterns for proper segment mapping

Original Interface:

module sevenseg(

input   logic           clk,

input   logic   [15:0]  displayed_number,  // Original: single 16-bit number

output  logic   [3:0]   Anode_Activate,

output  logic   [6:0]   LED_out

);

Modified Interface:

module sevenseg(

input   logic           clk,

input   logic   [3:0]   digit_0,    // Leftmost digit (direct input)

input   logic   [3:0]   digit_1,

input   logic   [3:0]   digit_2,

input   logic   [3:0]   digit_3,    // Rightmost digit

output  logic   [3:0]   Anode_Activate,

output  logic   [6:0]   LED_out

);

## 5.3 conditioner.sv - Button Debouncer

Source: Assignment 5 Starter Code (sevenSegmentRamAssignmentHandout)

Status: Modified

Modifications:

- Added one-pulse output generation (pulse signal)

- Added prev_conditioned register for edge detection

- Rising-edge pulse: pulse = conditionedSignal & ~prev_conditioned

Original Interface:

module conditioner(

input   logic   clk,

input   logic   buttonPress,

output  logic   conditionedSignal   // Only debounced output

);

Modified Interface:

module conditioner(

input  logic clk,

input  logic buttonPress,

output logic conditionedSignal,

output logic pulse              // Added: One-cycle pulse output

);

## 5.4 vga_controller.sv - VGA Timing Generator

Source: Professor's VGA Example Project (16_VGA_example.zip)

Status: Substantially Rewritten

Original Design: Used pixel strobe enable with 100MHz clock

Modified Design: Direct 25MHz pixel clock input with reset support

Key Changes:

- Replaced pixel strobe (i_PIX_STB) approach with dedicated 25MHz clock input

- Added asynchronous reset support

- Added video_on output signal for blanking control

- Changed sync signal polarity (active-low for standard VGA)

- Renamed signals to follow project conventions

Original VGA Timing Parameters (from professor's code):

// Professor's VGApulses module

c_HORIZ_ACTIVE = 10'd640,  c_HORIZ_FP = 10'd16,

c_HORIZ_SYN    = 10'd96,   c_HORIZ_BP = 10'd48,

c_VERT_ACTIVE  = 10'd480,  c_VERT_FP  = 10'd11,

c_VERT_SYN     = 10'd2,    c_VERT_BP  = 10'd32

Project's VGA Timing Parameters:

// vga_controller.sv

H_ACTIVE = 640, H_FRONT = 16, H_SYNC = 96, H_BACK = 48,

V_ACTIVE = 480, V_FRONT = 10, V_SYNC = 2,  V_BACK = 33

## 5.5 puzzle_rom.sv - File Loading Pattern

Source: RISC-V Single-Cycle Processor (riscv_single.sv)

Status: Pattern Adapted

The file loading approach using $readmemh/$fopen was inspired by the instruction memory (imem) and data memory (dmem) modules in the RISC-V processor:

RISC-V imem Pattern:

module imem(input  logic [31:0] a,

output logic [31:0] rd);

logic [31:0] RAM[63:0];

initial

$readmemh("Program_1.dat",RAM);

assign rd = RAM[a[31:2]]; // word aligned

endmodule

Project's puzzle_rom Adaptation:

module puzzle_rom(...);

logic [3:0] puzzle_data [0:5][0:80];

initial begin

file = $fopen("puzzles.mem", "r");

if (file == 0) begin

// Fallback to hardcoded puzzles

end else begin

// Read using $fgetc for character-by-character parsing

for (i = 0; i < 6; i++) begin

for (j = 0; j <= 80; j++) begin

char = $fgetc(file);

puzzle_data[i][j] = char - "0";

end

end

end

end

endmodule

## 5.6 PS/2 Keyboard Modules (Not Used)

Source: AA_ps2_keyboard.zip

Status: Attempted but not implemented

The project originally intended to use PS/2 keyboard input for number entry. The keyboard decoder and related modules were studied but ultimately not integrated due to hardware debugging difficulties. The project switched to button-only control instead.

Modules from AA_ps2_keyboard:

- PS2Decoder.sv - Shift register for PS/2 protocol

- keyboard_to_seven_seg.sv - Top-level keyboard demo

- seven_seg.sv - Alternative seven-segment implementation

# 6. References and External Sources

This section documents external sources and references used in the development of this project, beyond the professor-provided materials.

## 6.1 Bitmap Font Design

Files: digit_font.sv, letter_font.sv

Status: Original Design

The 8×12 pixel bitmap fonts for digits (1-9) and letters were designed by Gwendolyn Beecher specifically for this project. The font design was informed by studying common approaches to VGA text rendering in FPGA projects:

Reference Materials Consulted:

- Pong P. Chu, "FPGA Prototyping by VHDL Examples" - Chapter 13: VGA Controller II: Text

- dhepper/font8x8 (GitHub) - Public domain 8×8 monochrome bitmap fonts derived from IBM VGA fonts

- MadLittleMods/FP-V-GA-Text (GitHub) - VHDL VGA text display module

Implementation Approach:

The fonts use a case-statement lookup table pattern common in FPGA font ROMs. Each character is encoded as a nested case statement: the outer case selects the character, and the inner case selects the row (0-11). Each row returns an 8-bit pattern where '1' bits represent lit pixels.

// Example: Digit '1' bitmap (8×12 pixels)

4'd1: case(row)

0:  bits = 8'b00011000;  //    ██

1:  bits = 8'b00111000;  //   ███

2:  bits = 8'b00011000;  //    ██

...

10: bits = 8'b00111100;  //   ████

11: bits = 8'b00111100;  //   ████

endcase

## 6.2 VGA Timing Specification

The VGA timing parameters for 640×480 @ 60Hz are based on the VESA standard:

| Parameter | Horizontal | Vertical | Source |
| --- | --- | --- | --- |
| Active | 640 px | 480 lines | VESA Standard |
| Front Porch | 16 px | 10 lines | VESA Standard |
| Sync Pulse | 96 px | 2 lines | VESA Standard |
| Back Porch | 48 px | 33 lines | VESA Standard |
| Total | 800 px | 525 lines |  |

Pixel Clock: 25.175 MHz (approximated as 25 MHz using clock divider)

## 6.3 Basys3 Constraints File

File: Basys3_Master.xdc

Source: Digilent Reference Design

The XDC constraints file was sourced from Digilent's Basys3 reference materials and modified to uncomment only the pins used in this project (clock, buttons, switches, VGA, seven-segment display).

# 7. License

This project is licensed under the GNU General Public License v3.0.

GNU General Public License v3.0

Copyright (C) 2025 Gwendolyn Beecher, Constantine Hinds

This program is free software: you can redistribute it and/or modify it under the terms of the GNU General Public License as published by the Free Software Foundation, either version 3 of the License, or (at your option) any later version.

This program is distributed in the hope that it will be useful, but WITHOUT ANY WARRANTY; without even the implied warranty of MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU General Public License for more details.

You should have received a copy of the GNU General Public License along with this program. If not, see https://www.gnu.org/licenses/.

Permissions:

- Commercial use

- Modification

- Distribution

- Patent use

- Private use

Conditions:

- License and copyright notice must be included

- State changes made to the code

- Disclose source code

- Same license must be used for derivative works

# 8. Complete File Listing

| File | Lines | Description |
| --- | --- | --- |
| top.sv | 248 | Top-level integration module |
| sudoku_engine.sv | 364 | Game state machine and RAM control |
| shadow_register.sv | 301 | VGA display buffer |
| sudoku_draw.sv | 271 | VGA pixel rendering |
| letter_font.sv | 226 | 8×12 letter bitmaps |
| digit_font.sv | 169 | 8×12 digit bitmaps |
| sudoku_loader.sv | 162 | Puzzle loading FSM |
| sudoku_games.sv | 115 | Hardcoded puzzle data |
| sudoku_gameplay.sv | 110 | Cursor and number entry |
| puzzle_rom.sv | 95 | ROM with file loading |
| sudoku_checker.sv | 93 | Solution verification |
| sevenseg_sudoku.sv | 76 | Sudoku-specific 7-seg logic |
| sevenseg.sv | 76 | Low-level 7-seg driver |
| vga_controller.sv | 68 | VGA timing generation |
| conditioner.sv | 39 | Button debounce/one-pulse |
| clock_divider.sv | 27 | 100MHz → 25MHz |
| ram.sv | 17 | Professor-provided RAM |
| puzzles.mem | 7 | Puzzle data file |
| Basys3_Master.xdc | ~200 | Pin constraints |

— End of Document —